MLDL Saturn debugger v2.0 Sx/Gx

Jan Brittenson has graciously given me permission to post this modified

version of his MLDL library.

General changes:

- Version independant

- Uses HP mnemonics. Branch instructions are packed into one line, for

example:

?A=C	A,L06101	; packed GOYES L06101

?A=C	A,RTNYES	; packed RTNYES

- Uses RPL.TAB and DIS.TAB if present. CST 6 toggles tables on/off

- Faster recovery of MLDLpar

- Faster display routines

- Internal font to avoid garbled displays due to GC

- Removed CK1 to keep LastRomWord intact

- Added cycle counters

- Some minor fixes

Key changes:

[.]		Set mark. Was [*] previously

[ARG] [*]	Deep step with display update. If no ARG is given

uses ARG FFFFF.

[ARG] [/]	Shallow step with display update. If no ARG is given

uses ARG FFFFF.

[SIN]		Save registers. The registers are automatically saved

on start up.

[COS]		Swap saved registers with current ones.

[ARG] [SIN]	Set new cycle counter (usually [0] [SIN])

[ARG] [COS]	Set new total cycle counter (usually [0] [COS])

[CST] [7]	Toggle cycle counter/opcode display.

[CST] [6]	Toggle tables on/off

[ARG] [PRG] [N]	If ARG is zero breakpoint N is set to PC

Notes:

- MLDLpar size increased, purge old MLDLpar before installation.

- MLDL doesn't work from covered memory areas. If you have a GX

do not store MLDL to:

a) Card 2 if card 1 is present

b) Ports 3-33

- As a side effect of using a different assembler the command numbers

changed.

- The cycle counts are as explained in SASM.DOC.

Introduction:

#5F42 MLDB		; Start debugging GARBAGE

[+]			; Step into SAVPTR

[CST] [7]		; Toggle cycle counters on

; CD0EX takes 8 cycles, so far 14 cycles

; have been taken (GOSUBL)

[0] [SIN]		; Clear cycle counter 1. The two cycle

; counters are now different so both are

; displayed. Total cycle counter is still 14.

[0D] [*]		; Step 13 instructions with display update

; Execution ends at RTNCC

[+]			; Step RTNCC too. Now you can see SAVPTR takes

; 143 cycles to execute

[+]			; Step into GARBAGECOL

[F]			; Show instruction screen

[/]			; Start shallow step with display update

; Let it run for a while

[ON]			; Interrupt stepping

[A]			; Show registers

[COS]			; Back to start-up registers

[COS]			; Back to GARBAGECOL again

; This way you can check what registers have

; been used. For example before executing a

; subroutine press [SIN], execute subroutine

; with [-], then use [COS] to see changes.

[EVAL]			; Let execution finish

For those who didn't have MLDL before here's a document describing MLDL

version 1.02. The uuencoded binary is at the end.

Enjoy.

-----------------------CLIP-------------------------------------------

1.

MLDL 1.02, APRIL 1991

---------------------

The MLDL is a Machine Language Development Library for the HP-48SX,

and as such is primarily intended for HP-48 Machine-Language

Programmers familiar with the following:

o  The Saturn instruction set

o  Assembler programming

o  Machine code debugging

o  RPL internals - GC, system RPL, PMC

1.

INSTALLING THE LIBRARY

----------------------

The MLDL consists of five named commands, and has been given the

number 1092 decimal (hexadecimal 444). To install it, transfer the

binary file to your calculator and store it in a suitable port. Turn

the calculator off, then back on, and go to the directory where you

wish to attach it - HOME is fine. Enter :p:1092 where `p' is the port

you stored it in, then ATTACH. To remove it, go to the directory where

you previously attached it, enter `:p:1092', press DUP DETACH, then

PURGE.  The library does not automatically attach.

Installation example, assuming I/O has been properly set up:

HOME

'MLDL' KGET

MLDL :0:1092 STO

(Turn the calculator OFF, then back ON)

:0:1092 ATTACH

Removal example:

HOME

:0:1092 ENTER DUP

DETACH

PURGE

The display should briefly flicker as you purge the library from

the port.

3.

LIBRARY COMMANDS

----------------

[ABOUTMLDL]

Displays version and copyright.

[MLDB]

Saturn ML Debugger. This description covers the MLDB local mode. It

can also operate in one of two server modes: interactive or protocol,

described under MLDB Server Modes below. In Local Mode, the HP-48

display and keyboard, are used to control the debugger. In Server

Mode, control is maintained via the serial port, either from a dumb

ASCII terminal (Interactive Mode), or a dedicated front-end (Protocol

Mode). To assert that MLDB is in Local Mode when invoked, clear user

flags 32 and 33:

32 33 CF CF

MLDB allows you single-step ML programs, as well as examine

registers and memory contents. Since it single-steps ML, it is not

useful for debugging RPL code - unless you wish to follow your RPL

thread on an ML level. When invoked, an argument in expected in level 1:

Code object

Halts before the first instruction of the code object.

Both the PC and A registers are set to the address of the

first instruction.

XLIB

The XLIB must be a code object, which becomes the

argument.

Binary integer

Treats the binary integer as the address of a prefixed

machine code routine (PMC). Halts at the first instruction

of the PMC. PMCs consist of a 5-nybble pointer to the first

instruction which is usually, but not always, the address

of the PMC plus 5.

@#3a81 (True)

Any object other than @#3a81

Ignored - the debugger returns immediately. In the

future, @#3a81 (True) will cause the debugger to use the

next object in sequence. It will be possible to include a

call to MLDB and precede it with a test to determine

whether to actually debug or run free. A user flag test,

for instance.

4.

Local Mode MLDB uses PICT (the graphical display) to present

information. Since all information will not fit on one display at

once, it has been divided into 7 screens. Only one screen is active at

any one time. Switching between screens is done by means of the 6 menu

keys, here referred to as A-F, which correspond to screen 1 through 6

respectively, and the MTH key, which corresponds to screen 7.

The sample screens 1-7 below are from the first instruction of the

PMC at #59CC, and can be reasonably reproduced by:

#59CC MLDB

Screen 1  - General CPU State (key [A])

Mnemonic............................ |CALL.4 #0679B         |

Opcode.............................. | 8E4CD0 	       |

PC, P, Carry, Hex/Dec mode, ST...... |@:059D1 P:0 CH ST:218 |

A.A and C.A......................... |A:000CC C:77794       |

B.A, D.A, and HST................... |B:729A9 D:00F96 HST:2 |

D0 and 6 bytes @D0.................. |D0:409C1/9540A8240BC9 |

D1 and 6 bytes @D1.................. |D1:77799/000000000000 |

Top 3 levels of RSTK................ |RST:00000:00000:00000 |

Screen 2  - Arithmetic registers (key [B])

Mnemonic............................ |CALL.4 #0679B         |

Opcode.............................. | 8E4CD0 	       |

PC, P, Carry, Hex/Dec mode, ST...... |@:059D1 P:0 CH ST:218 |

Register A.......................... |A:00000005444000CC    |

Register B.......................... |B:000000000007611E    |

Register C.......................... |C:000000000007792C    |

Register D.......................... |D:00000000000004D0    |

Top 3 levels of RSTK................ |RST:00000:00000:00000 |

Screen 3  - Data registers (key [C])

Mnemonic............................ |CALL.4 #0679B         |

Opcode.............................. | 8E4CD0 	       |

PC, P, Carry, Hex/Dec mode, ST...... |@:059D1 P:0 CH ST:218 |

Register R0......................... |R0:053385D439800040   |

Register R1......................... |R1:00000005444059D1   |

Register R2......................... |R2:0000000000075BC1   |

Register R3......................... |R3:0000000544402E92   |

Register R4......................... |R4:00015075A6375AA1   |

Top 3 levels of RSTK................ |RST:00000:00000:00000 |

5.

Screen 4  - Return stack (key [D])

Mnemonic............................ |CALL.4 #0679B         |

Opcode.............................. | 8E4CD0 	       |

PC, P, Carry, Hex/Dec mode, ST...... |@:059D1 P:0 CH ST:218 |

| 		       |

RSTK levels 0 and 4................. |RST0:00000 RST4:00000 |

RSTK levels 1 and 5................. |RST1:00000 RST5:00000 |

RSTK levels 2 and 6................. |RST2:00000 RST6:00000 |

RSTK levels 3 and 7................. |RST3:00000 RST7:00000 |

Screen 5  - Memory dump (key [E])

Locations 59A0-59AF................. |059A0:56113680913420CC|

Locations 59B0-59BF................. |059B0:4E0156716FCC56FD|

Locations 59C0-59CF................. |059C0:015B38D5E0101D95|

Locations 59D0-59DF................. |059D0:08E4CD08E46C0101|

Locations 59E0-59EF................. |059E0:D230574911191443|

Locations 59F0-59FF................. |059F0:4E4A201101311456|

Locations 5A00-5A0F................. |05A00:12280A50143174E7|

Locations 5A10-5A1F................. |05A10:8E58D01311741431|

The current location is indicated by an inverse digit.

Screen 6  - ML Instruction Stream (key [F])

PC, P, Carry, Hex/Dec mode, ST...... |@:059D1 P:0 CH ST:218 |

Next 7 instructions................. |#CALL.4##0679B########|

| CALL.4 #06641	       |

| MOVE.W A,R1	       |

| CLR.A C	       |

| MOVE.P1 #5,C	       |

| CALL.3 #05B7D	       |

| MOVE.W R1,C	       |

The next instruction is the one displayed in reverse. Currently,

it will always appear at the top. (Here somewhat crudely

illustrated with the # signs.)

Screen 7  - Breakpoint Table Screen (key MTH)

Breakpoint #1....................... | 1:6100 +02	       |

Breakpoint #2....................... | 2:6104 -02	       |

Breakpoint #3....................... | 3:613A  00	       |

Breakpoints #4-#8: not used......... | 4:0000  00	       |

| 5:0000  00	       |

| 6:0000  00	       |

| 7:0000  00	       |

| 8:0000  00	       |

Any breakpoints at the current location are displayed in reverse.

6.

The Local Mode MLDB keyboard

----------------------------

A number of keys will accept an argument, referred to as ARG. To

enter ARG, press [0] (the zero key) followed by a hex integer, which

will make up ARG. It is restricted to 5 hex digits (20 bits). As [0]

is pressed, the bottom display line, regardless of screen currently

active, becomes an ARG entry line:

|ARG:00000             |

This reformat remains in effect until a non-hex key is pressed, at

which point that key is passed ARG. Some keys behave differently

depending on whether ARG was present - [DEL], for instance, does

nothing, effectively cancelling ARG. The backarrow, [<==], divides the

argument by 16, in effect shifting it right one digit. The argument is

always a 20-bit unsigned integer. The digits A-F are found on the menu

keys.  The [+/-] key negates (2's complement) the argument.

Example, ARG entry:

Keys		Comments	Bottom line following keys

0		Start entry	|ARG:00000             |

5		Digit 5		|ARG:00005             |

7		Digit 7		|ARG:00057             |

[<==]		Shift right	|ARG:00005             |

9		Digit 9		|ARG:00059             |

D1		Digits D, 1	|ARG:059D1             |

[+/-]		Negate		|ARG:FA62F             |

000		Three 0s	|ARG:2F000             |

[<==]		Shift right	|ARG:02F00             |

[DEL]		Cancel ARG

7.

Moving about is done with the arrow keys and the NXT key - notice

that none of these keys actually execute the previous instruction,

only the PC is affected:

[up]	Decrement PC by 16 (or 16*ARG)

[down]	Increment PC by 16 (or 16*ARG)

[left]	Decrement PC by 1 (or ARG)

[right]	Increment PC by 1 (or ARG)

[NXT]	Move to next instruction (or ARG instructions forward)

In addition, there is a mark:

[x]	(Mulitplication key)

Set mark to ARG, if present. Otherwise the MARK

is set to the current PC.

[+/-]	Swaps the PC and the mark.

Example, Moving About:

Keys		Comments	Display following keys

059D1[ENTER]	PC=#059D1h	|@:059D1 P:0 CH ST:218 |

|#CALL.4##0679B########|

| CALL.4 #06641        |

| MOVE.W A,R1          |

| CLR.A C              |

| MOVE.P1 #5,C         |

| CALL.3 #05B7D        |

| MOVE.W R1,C          |

04[right]	PC=PC+4		|@:059D5 P:0 CH ST:218 |

|#CLR.A#A##############|

| CALL.4 #06641        |

| MOVE.W A,R1          |

| CLR.A C              |

| MOVE.P1 #5,C         |

| CALL.3 #05B7D        |

| MOVE.W R1,C          |

059D1[ENTER]	PC=#059D1	|059A0:56113680913420CC|

[E]		Memory dump,	|059B0:4E0156716FCC56FD|

screen 5	|059C0:015B38D5E0101D95|

|059D0:0#E4CD08E46C0101|

|059E0:D230574911191443|

|059F0:4E4A201101311456|

|05A00:12280A50143174E7|

|05A10:8E58D01311741431|

8.

[up]		PC=PC-#10h	|05990:3A6E80D0F40D4F01|

|059A0:56113680913420CC|

|059B0:4E0156716FCC56FD|

|059C0:0#5B38D5E0101D95|

|059D0:08E4CD08E46C0101|

|059E0:D230574911191443|

|059F0:4E4A201101311456|

|05A00:12280A50143174E7|

059D1[ENTER]	Back to #59D1h	|@:059D1 P:0 CH ST:218 |

[F]		ML instructions	|#CALL.4##679B#########|

| CALL.4 #06641        |

| MOVE.W A,R1          |

| CLR.A C              |

| MOVE.P1 #5,C         |

| CALL.3 #05B7D        |

| MOVE.W R1,C          |

[NXT]		Forward one	|@:059D7 P:0 CH ST:218 |

instruction	|#CALL.4##06641########|

| MOVE.W A,R1          |

| CLR.A C              |

| MOVE.P1 #5,C         |

| CALL.3 #05B7D        |

| MOVE.W R1,C          |

| MOVE.A C,@D0	       |

[x]		Set mark	|@:059D7 P:0 CH ST:218 |

03[NXT]		Forward 3	|#MOVE.P1##5,C#########|

instructions	| CALL.3 #05B7D        |

| MOVE.W R1,C          |

| MOVE.A C,@D0         |

| MOVE.P5 #02A4E,C     |

| MOVE.W R0,A          |

| MOVE.A A,D1          |

[+/-]		Toggle mark,	|@:059D7 P:0 CH ST:218 |

PC		|#CALL.4##06641########|

| MOVE.W A,R1          |

| CLR.A C              |

| MOVE.P1 #5,C         |

| CALL.3 #05B7D        |

| MOVE.W R1,C          |

| MOVE.A C,@D0         |

The arrow keys are most useful for moving around in the memory

dump, but can also be used to arbitrarily increment and decrement the

PC to shift the instruction stream by single nybbles. The mark

commands are useful for temporarily remembering an address.

8.

There are three ways of aborting your program, and seubsequently

leaving the debugger:

[<==]	(Delete key)

Exits.

[DEL]	Restores system registers to the state they were in

when the debugger was entered. This is useful if you

would need to exit in the middle of your program and

the system registers contain random data. It does

nothing if ARG was supplied, in effect acting as an

ARG cancellation key.

[1/X] [1/X]

Panic exit. If you know the machine will crash when

you exit, such as could be the case if memory has been

reconfigured. Resets the calculator, which causes the

MLDL to become detached. Follow the procedure

described in the first section of this document to

reattach.

Four keys are used to execute your program:

[+]	Single-steps one (or ARG) instruction(s), pointed to

by the PC. Follows CALLs. If ARG, then also checks for

breakpoints.

[-]	Same as [+], but does not follows CALLs; instead, the

program halts when it returns from the CALL.

[.]	Redraws the display but otherwise does nothing.

[EVAL]	Continue execution, run until program completes,

breakpoint is reached, or [-] CALL execution

completes.

9.

The MLDB maintains a breakpoint table of eight slots. Each slot

consists of an address and a counter, both of which are displayed when

the MTH key is pressed (see below). The address is 20 bits and the

counter is eight bits. In addition, each slot holds the breakpoint

instruction (a CALL instruction) to be inserted into the program when

running it. Breakpoints only work in RAM, although they can be set in

ROM and triggered by multi-instruction single-steps. When a breakpoint

is triggered, its counter is incremented; if after the increment it is

negative (80-FF), the program will continue. Setting a negative

counter is therefore a way of instruction MLDB to "ignore this

breakpoint N times" where N is in the range 01-7F hexadecimal. If the

counter is positive, the program is halted. Positive counters indicate

how many times the program has halted at a specific breakpoint.

Example, Program Completion:

Start with the stack display, HEX mode, in the MLDL library

menu. Only the relevant lines of the stack display are

included in the example below. #3244h is DROP.

Keys		Comments	Display following keys

HEX STD		Set up modes

2 1 #3244	Enter two	|3:                   2|

[ENTER]		arguments for	|2:                   1|

DROP, and the	|1:             # 3244h|

of the DROP

PMC

[MLDB]		Invoke MLDB on  |@:03249 P:0  H ST:218 |

[F]		the DROP PMC	|#ADD.A##5,D1##########|

and choose the	| INC.A D              |

instruction	| MOVE.A @D0,A         |

screen		| ADD.A #5,D0          |

| JUMP.A @A            |

| MOVE.A C,B           |

| MOVE.1 #3,P          |

The DROP PMC consists of only the first 5 instructions.

The JUMP @A will cause the program to reach completion.

[EVAL]		Allow program	|1:                   2|

to run freely

10.

Breakpoints are accessible via three keys:

[MTH]	Switch to breakpoint table screen. Any entry at the

current PC will be displayed in reverse video.

[PRG]	Set breakpoint. Waits for a further key, 1-8, which

specifies the breakpoint to set. ARG is the address

the breakpoint is set to. If no ARG is entered, then

the breakpoint is cleared (its address and counter are

set to 00000 and 00, respectively).

[STO]	Set breakpoint counter. Waits for a further key, 1-8,

which specifies the breakpoint whose counter is to be

set. ARG is the new counter value. The counter is

cleared if no ARG is entered. Only the low 8 bits of

ARG are used.

Example, Breakpoints. For an illustration on how to enter ARG, see

the "ARG entry" example above.

Keys		Comments	Display following keys

059D1[ENTER]	PC=59D1		|CALL.4 #0679B         |

[A]		Screen 1	| 8E4CD0               |

|@:059D1 P:0 CH ST:218 |

|A:000CC C:77794       |

|B:729A9 D:00F96 HST:2 |

|D0:409C1/9540A8240BC9 |

|D1:77799/000000000000 |

|RST:00000:00000:00000 |

[MTH]		Breakpoint	| 1:00000  00	       |

table screen	| 2:00000  00	       |

| 3:00000  00	       |

| 4:00000  00	       |

| 5:00000  00	       |

| 6:00000  00	       |

| 7:00000  00	       |

| 8:00000  00	       |

059D1[PRG]1	Set		|#1:059D1##00##########|

breakpoint 1	| 2:00000  00	       |

at #59D1h	| 3:00000  00	       |

| 4:00000  00	       |

| 5:00000  00	       |

| 6:00000  00	       |

| 7:00000  00	       |

| 8:00000  00	       |

The inverse bar in the display above indicates that the

breakpoint is set at the current PC.

11.

[F]		Examine	next	|@:059D1 P:0 CH ST:218 |

7 instructions	| CALL.4 #0679B	       |

| CALL.4 #06641	       |

| MOVE.W A,R1	       |

| CLR.A C	       |

| MOVE.P1 #5,C	       |

| CALL.3 #05B7D	       |

| MOVE.W R1,C	       |

[MTH]		Back to breakpoint

table screen

06641[PRG]5	Set		|#1:059D1##00##########|

breakpoint 5	| 2:00000  00	       |

at #6641h	| 3:00000  00	       |

| 4:00000  00	       |

| 5:06641  00	       |

| 6:00000  00	       |

| 7:00000  00	       |

| 8:00000  00	       |

040[+]		Single-step 64	| 1:059D1  00          |

instructions	| 2:00000  00          |

| 3:00000  00          |

| 4:00000  00          |

|#5:06641#+01##########|

| 6:00000  00          |

| 7:00000  00          |

| 8:00000  00          |

We never really got as far as 64 instructions. Instead we

ran into the breakpoint at #6641h and stopped there. The

breakpoint table above tells us why we stopped, and that it is

our first stop here. Setting breakpoints and single-stepping a

large number of instructions is the only way to use

breakpoints in ROM.

[STO]5		Clear counter	| 1:059D1  00          |

of breakpoint 5	| 2:00000  00          |

| 3:00000  00          |

| 4:00000  00          |

|#5:06641##00##########|

| 6:00000  00          |

| 7:00000  00          |

| 8:00000  00          |

12.

A breakpoint is a CALL.A to an entry in the MLDB. When the

breakpoint is reached, control is transfered to the breakpoint trap

handler in the MLDB. It finds the location of the breakpoint on the

return stack (RSTK), looks it up in the breakpoint table, and proceeds

as outlined above. The CALL.A instruction uses 7 nybbles of memory.

Therefore, problems can arise when the program contains a sequence

like this:

point:	brcc	foo		; 3 nybbles

inc.a	c		; 2 nybbles

foo:	inc.a	c		; 2 nybbles

dec.a	a		; 2 nybbles

Picture what would happen if a breakpoint is set at `point.' It

uses up 7 nybbles of memory, and thus overwrites the BRCC, subsequent

INC, and the INC at `foo.' Assume another piece of the program makes a

jump to `foo.' This will result in a jump into the last two nybbles of

the breakpoint instruction! The program is bound to break - if we're

lucky this means mysterious results - if we're unlucky, we're going

down in flames (metaphorically speaking).

During a [+] single-step of multiple instructions (actually, any

single-step with an ARG will do), the breakpoints are never inserted

into the program. Instead, each consecutive PC is matched against the

breakpoint table, and if a breakpoint is set at exactly that address,

the single-stepping will stop. This usage of breakpoints is entirely

safe and works under all conditions. It can, of course, be used for

ROM as well as RAM programs. The drawback is execution speed, although

the [-] stepping variant is usually faster than the [+] variant.

Graphics and other CPU-heavy applications take seemingly forever to

run. There is no simple solution - you must take the 7-nybble limit

into consideration when writing such programs. Insert NOPs at places

where you know you will want to insert breakpoints.

The [-] single-step does not follow CALLs. It leaves control to the

subroutine called and lets it run freely until it returns to the

caller, which is the MLDB. It then proceeds with the next instruction

in sequence, the one following the CALL. Time-consuming functions like

graphics routines can be placed in separately debugged and verified

subroutines, and the main program calling these routines debugged with

[-]. This will generally give acceptably quick response.

When during [-] step single-a subroutine that is allowed to run free until

it returns, encounters a breakpoint, the program halts. The return

address _into_MLDB_ will be on top of the return stack. Pressing

[EVAL] to continue at this point will cause the subroutine to continue

running free, until it again reaches a breakpoint, or returns, at

which point it returns to MLDB and continues its [-] single-step. The

subroutine can also be single-stepped when it has run into a

breakpoint.

13.

MLDB Server Modes

-----------------

User flags 32 and 33 control the MLDB mode:

Server,	 Server,

Local	Interactive	Protocol

------------------------------------------

32	Clear	    Set		   Set

33	Clear	   Clear	   Set

The normal mode of operation, as described in the previous

sections, is Local mode. The other two modes are referred to as Server

Modes, and are the Interactive Mode - commands (see Server Commands,

below) are entered on a dumb ASCII terminal or emulator, with full

editing (see Server Command Entry, below) - and Protocol Mode, in

which the same commands are accepted as in Interactive Mode, except no

prompts are printed and commands are not echoed during reception. The

Protocol Mode is intended for specialized MLDL Server Front-End

Software (FES) - refer to the manual of the Server FES in question for

details.

14.

Server Mode Commands

--------------------

Commands can be entered in response to the ``*'' prompt in

Interactive Mode, or whenever suitable - they will be processed

sequentially - when in Protocol Mode. Generally, excessive input is

ignored, as are unrecognized commands or commands with invalid

arguments. Commands of up to 40 characters can be entered.

= <addr>	Set the PC to <addr>.

<addr> is in the range 00000-FFFFF.

+ <offs>	Add <offs> to PC.

<offs> is in the range 00000-FFFFF.

- <offs>	Subtract <offs> from PC.

<offs> is in the range 00000-FFFFF.

n [<n>]		Advance PC forward <n> instructions. If no <n>

is supplied, the PC is advanced one

instruction.

s [<n>]		Single-step <n> instructions. Same as the [+]

key in Local Mode. If no <n> is entered, one

instruction is stepped.

S [<n>]		Single-step <n> instrutions, but don't follow

CALLs. Same as the [-] key in Local Mode. If

no <n> is entered, one instruction or CALL is

is stepped.

c		Continue free-run execution until the program

completes or a breakpoint is encountered. When

the program completes, ``Exit'' is printed and

MLDB exits.

t		Terminate. Exit with current registers.

T		Terminate. Exit with system registers set up

exactly like when MLDB was first invoked.

R		Reset.

r		Print registers. The HEX/DEC mode is printed

as ``HD:0'' or ``HD:1''. A zero means that HEX

mode is active, a one means that DEC mode is

active.

i [<n>] [<addr>]

Print instructions. <n> instructions are

printed, starting at <addr>. The first

argument is always <n> and the second is

<addr>. If no <addr> is entered, the current

PC is assumed. If <n> isn't entered, one - the

next - instruction is printed. <n> and <addr>

are both in the range 00000-FFFFF.

15.

x [<n>] [<addr>]

Print memory contents. <n> words of 16 nybbles

are printed, starting at <addr>, each on a

separate line. The first argument is always

<n> and the second is <addr>. If no <addr> is

entered, the current PC is assumed. If <n>

isn't entered, one - the next - word is

printed. <n> and <addr> are both in the range

00000-FFFFF.

z		Print the return stack - the RSTK.

db <n> [<addr>]	Set breakpoint <n> at <addr>.  If <addr> is

absent, the breakpoint is cleared. <addr> is

in the range 00000-FFFFF. <n> must be in the

range 1-8.

hb <n> [<cntr>]	Set breakpoint <n> counter to <cntr>. If

<cntr> is absent, the counter is set to 00.

<cntr> is in the range 00-FF. <n> must be in

the range 1-8.

lb		List breakpoints.

16.

Server Command Entry

--------------------

In both Interactive and Protocol Modes, input can be edited,

although no echo or response can be detected in Protocol Mode. The

following table may be of help when you try to locate the editing keys

on your keyboard:

Backspace, Delete, 	Erase the last character entered.

or Rubout

Control-W		Erase the last word entered.

Control-U or		Erase entire line.

Control-X

Control-R		Rewrite input.

Return, Enter, or	Execute command entered.

Control-M

17.

Exampe, An Interactive Mode Session

-----------------------------------

We invoke MLDB with #3223, the address of the SWAP PMC. We pass two

arguments to SWAP - 1 and 2. First, we are greeted with a header.

MLDL 1.02

Copyright (c) 1991 Jan Brittenson

Let's first take a look at the SWAP PMC:

* i 9

MOVE.A @D1,C

ADD.A #5,D1

MOVE.A @D1,A

MOVE.A C,@D1

SUB.A #5,D1

MOVE.A A,@D1

MOVE.A @D0,A

ADD.A #5,D0

JUMP.A @A

Then the registers:

* r

CY:0

P:0

PC:03228

A:0000000644403223

B:0960000000074FB3

C:0000000000075FBB

D:0000000000000335

R0:000000000007BCA5

R1:0000000644403228

R2:00000000000505C6

R3:0000000644400001

R4:00015074EE274F20

D0:7C1A5

D1:75FC0

ST:000

HST:2

HD:0

So stack level 1 is at 75FC0.

* x 1 75fc0

75FC0:ED2A29C2A2000000

Which is object 2A2DE.

* x 2 2a2de

2A2DE:3392000000000000

2A2EE:0002033920000000

Which is a real (type prefix 2933), the constant 2. Let's step a

few instructions.

18.

* i

MOVE.A @D1,C

* s

* i

ADD.A #5,D1

* s

* i

MOVE.A @D1,A

* s

* i

MOVE.A C,@D1

* s

* i

SUB.A #5,D1

* s

* i

MOVE.A A,@D1

* s

Now, what do we have left?

* i 3

MOVE.A @D0,A

ADD.A #5,D0

JUMP.A @A

We step a goodly chunk of instructions. That way we can be sure

that the program completes.

* s 100

Exit

We're done. The HP-48 stack now reads "2" in level 2, and "1" in

level 1.

19.

Some system considerations

--------------------------

The debugger has been designed explicitly so that it will not alter

any static system data or depend on the precise machine configuration.

The only system data it modifies is the keyboard buffer, since it

relies on the system to respond to the keyboard interrupt and manage

the buffer.  Testing has shown that interfering with this results in

poor reliability. Still, there are two instructions the debugger will

refuse to single-step:

RESET

The effect of this would be the same as ON-C.

CLRB #F,ST

Executing this instruction would lock up your calculator

since it would disable all I/O most notably the keyboard.

Apart from the aspects outlined above and some system RPL code to

do argument type checking, the debugger is self-contained.

20.

A word of caution

-----------------

The [-] key lets you complete an entire CALL. But beware: the

return stack is replaced by one that will cause the called routine to

return to the debugger. Therefore the routine called cannot rely on

specific return stack contents, or remove return addresses from the

stack, either of which will invariably result in a system crash.

Fortunately, this appears not to be standard practice in the HP-48

ROM.

Despite the effort put into avoiding system collisions, the HP-48

will still remain a largely unprotected system where the debugger - or

other parts of the MLDL - can still be overwritten, which may result

in memory loss. Since the MLDL is stored as a library in a port, it is

not susceptible to general memory allocation, such as dynamic memory

allocation or system display GROB allocations.  On the other hand,

storing another library in the same port may cause it to move -

extreme caution is therefore advisable when single-stepping PMC that

alters stored port data. If you are stepping a port store block copy

by means of either [+] or [-], then the MLDL may be overwritten. If

you're using [-] to call a block copy routine within a similar

context, then the library may have moved and the subsequent return

will cause a system crash and possibly memory corruption.

Single-stepping a machine code program is in no way less dangerous

than allowing it to run uncontrolled. It merely gives you some control

over what happens between instructions. It can even be more dangerous;

hardware may break if left in certain configurations for a period of

time, which may be a fraction of a second. So if you are going to

single-step parts of the system ROM, you should be aware of this risk.

In no way will the author or distributors of the MLDL accept any

responsibility or liability for such damage, regardless of its nature

and extent.

The MLDL is distributed in the hope that it will be useful, but

WITHOUT ANY WARRANTY; without even the implied warranty of

MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE.

21.

[MLPR]

Disassembles and prints ML programs. Accepts the same arguments as

MLDB, except @#3a81 and #3ac0 which are not recognized. The program is

printed to the current print device - IR or Serial. In case of

problems, first check your PRTPAR - see pages 602-611 in your Owner's

Manual Volume II. Page 611 explicitly describes the PRTPAR. Each line

consists of a mnemonic preceded by the address. No opcode is included,

since it normally is of low interest. Use ML1 (described below) to

build your own custom disassembler.

[ML1]

Disassembles one ML instruction; allows you to build your own

disassemblers with special-purpose user interfaces. It takes a binary

integer in level one, and returns two values; in level 2 the mnemonic

form preceded by the address, and in level one the address of the next

instruction. Thus it is a simple task to make a number of consequtive

calls to ML1. MLOPC can be used to extract the opcode as a string of

hexadecimal digits. Extract The mnemonic from the string is trivial,

since it will always be of the form:

"xxxxx: m"

where xxxxx is a five-digit address followed by a colon and a blank.

The last part of the string, m, is the mnemonic.

[MLOPC]

Returns opcode as a string of hexadecimal digits. Expects two

binary integers - the first address in level 2 and the final address

plus one in level 1. Useful for creating the opcode field in a custom

disassembler. Example, MLOPC:

#59D1 #59D7 MLOPC  -->  "8E4CD0"

The opcode size is limited to 255 digits.

22.

MLDL COMMAND SUMMARY

--------------------

|-----------------------------------------------------------------------|

|									|

| Version	Returns version and copyright strings			|

|									|

|				--> "version" "copyright"		|

|									|

|-----------------------------------------------------------------------|

|									|

| MLDB		Saturn Machine Language Debugger			|

|									|

|			obj	--> any1 ... anyN			|

|									|

|-----------------------------------------------------------------------|

|									|

| MLPR		Print Machine Language Program				|

|									|

|			obj	--> obj					|

|									|

|-----------------------------------------------------------------------|

|									|

| ML1		Single-Instruction Machine Language Disassembler	|

|									|

|		      #address	--> "instruction" #next			|

|									|

|-----------------------------------------------------------------------|

|									|

| MLOPC		Machine Language Instruction Opcode			|

|									|

|	   #address1 #address2	--> "hex digits"			|

|									|

|-----------------------------------------------------------------------+

23.

MLDB LOCAL MODE KEYBOARD SUMMARY

--------------------------------

Screens		No ARG			ARG

[A]		General CPU State

[B]		Arithmetic registers

[C]		Data registers

[D]		Return stack (RSTK)

[E]		Memory dump

[F]		Instruction stream

[MTH]		Breakpoint table	Breakpoint table

ARG entry

[0]		Begins ARG entry

0-9, A-F	Hex digits

[+/-]		2's complement

[<==]		Shift right one digit

[DEL]		Cancel

Moving around

[ENTER]		Ignored			PC=ARG

[left]		PC=PC-1			PC=PC-ARG

[right]		PC=PC+1			PC=PC+ARG

[up]		PC=PC-16		PC=PC-16*ARG

[down]		PC=PC+16		PC=PC+16*ARG

[x]		MARK=PC			MARK=ARG

[+/-]		MARK <-> PC		MARK <-> PC

[NXT]		Advance one 		Advance ARG instructions

instruction

Breakpoints (b = 1 - 8)

[MTH]		Breakpoint table 	Breakpoint table screen

screen

[PRG]b		Clear breakpoint b	Set breakpoint b at ARG

[STO]b		Clear breakpoint b	Set breakpoint b counter

counter			to ARG

Program execution

[+]		Single-step one		Single-step ARG instructions

instruction

[-]		Single-step one		Single-step ARG instructions,

instruction, do		do not follow CALLs.

not follow CALLs

[EVAL]		Let program run until	Ignored

completed, or until

breakpoint

24.

MLDL XLIB TABLE

---------------

The command numbers are subject to change at random and without

prior notice. The library number is 444 hexadecimal, which is 1092

decimal.

Command		Number

----------------------

Version		1092 0

MLDB		1092 1

MLPR		1092 2

ML1		1092 3

MLOPC		1092 4

--->	Mika Heiskanen, mheiskan@delta.hut.fi

-=- Dis file passed thru -=-

+  /               .                              /     .    .

/|     *                -o-    .      ' '      /|___.._   /   +

//____  ____...__   , ______      +    |'|'    /        \ /|    ___  ___  .  '

(      \/        /  / /      \        __||||__  )   .     \\ \__/   \/   \    |

\_____  ______  \ /\ }  __   }   .  /  .     }/    |\     \\             } _/|

{  }  /  \ // //  /  \ /      /   |\   /{    /  \     }\   ___/\___/_/  |

+ |  | /|   |/ //   \_  |  '   {    \|  {  \  {    \   /  }  \ .   +  )   |

.     \  /| \___/  |\    _) .  |    \    \   \  \  \    \ {  /    \___.._/    /

/  \\   __   //   /     /|  +  }   /\   \  \  \   /  \{            \   /

|  | |  } }  }\  (__   / /    /   / /    \  }  \ /   | \     _..___ } /

+   / /  ) / /  / /     \_( /    /    \/      }/   / \   |  }   {   / |/ /

|/  /_/ /  / {   __    / ' . \     \     / |  /   \_/  /     \_/  / /

.    /  //  (  (   \ /  \__/ \|/   \     .   /  | /     =  {     _   .  /

.  |/    \ |  . \|     - -*- -  \___..._/   |/   .  _   \___/ \    /   *

/      \|     \  .    /|\       ||||     /                  \  /

+  '    .   '            .   .      '|'|    '        +  BBS .    \/

+             '           .  .      .                    .

===============================================================================

Sector 0ne/Eko/FFT/Equinox/Furtive/Eagles WHQ -~- <> -~- Control Team French HQ

==============================> HP 48 World Base <=============================

-=-   P/H/A/C, HP48, Falcon, ST, Amiga, PC warez, hints, Chats, and more.   -=-

==================================> Ring now : <===============================

-=-  [ + 3 3 ] - ( 1 ) 4 9 - 8 8 7 - 6 9 1 -=- 1 Go 0/L -=- Paris / France  -=-

===============================================================================